Hydra-NL Water Level Extraction

- Hydra-NL desktop app installation. Contant Karolina Wojciechowska for installation details and to get last  Hydra-NL version.

- Open Hydra-NL in Beoordelingsmodus.

- Explore and load a database of a region of interest by selecting Randvoorwaarden>Beheren>Toevogen

A database of the area of interest in .sqlite  format downloaded from STACK - Shared files (stackstorage.com) or STACK - Shared files (stackstorage.com) can then be added. In this case, WBI2017_Waddenzee_Oost_6-7_v03.sqlite. Visit Map images (waterveiligheidsportaal.nl) to know more about the areas of interest names.

If added successfully, the database can be visualized in the Randvoorwaarden window.

- Visualize area of interest by selecting the database in the map using the scroll down area of interest menu.

- Select the point of interest to generate exceedance water level. Move, zoom in and select with the top left home screen tools  .

- Calculate exceedance water level by selecting Berekening>Start dijkvakberekening. Assing a name for the calculation in Naam berekening and for the saved excel file in Naam overzichtbestand. In this case, excedance_water_level_6-7. Ensure to check Resultaren naar overzichtbestand schrijven box

To define exceedance frequencies go to Parameters in the Start dijkvakberekening window and change Frequenties settings. Frequencie years are only available for 10 – 1000000 range.

Press OK in the parameters window, and then Start in the Start dijkvakberekening window to compute excedance water levels for the selected point of interest.